cDataSend[0] = BMP280_READ_DIG_TX;

twi_master_transmit(SLAVEADRESS, cDataSend, 1, 0);	                                                   // Write slave adress, first register adress, length of data, no repeated start

twi_master_receive(SLAVEADRESS, cDataReceive, 6);	                                                   // Receive data from bmp280, write data in to cDatareceive, 6 repeated starts

i32Dig_T1 = (cDataReceive[0] | (cDataReceive[1] << 8)) & 0x0000FFFF;

i32Dig_T2 = (cDataReceive[2] | (cDataReceive[3] << 8)) & 0x0000FFFF;

i32Dig_T3 = (cDataReceive[4] | (cDataReceive[5] << 8)) & 0x0000FFFF;

cDataSend[0] = BMP280_READ_ADC_T;

twi_master_transmit(SLAVEADRESS, cDataSend, 1, 0);

twi_master_receive(SLAVEADRESS, cDataReceive, 3);

cDataReceive[2] = cDataReceive[2] & 0xF0;

i32Adc_T = ((cDataReceive[2] >> 4) | (cDataReceive[1] << 4) | (cDataReceive[0] << 12));

//Conversion to variable

#define BMP280_RESET				0xE0

#define BMP280_RESET_ACTIVATE			0xB6

#define BMP280_CTRL_MEAS			0xF4

#define BMP280_CTRL_MEAS_DATA			0x2F

#define BMP280_CONFIG				0xF5

#define BMP280_CONFIG_DATA			0x02

#define BMP280_READ_ADC_T			0xFA

#define BMP280_READ_DIG_TX			0x88

#define BMP280_READ_ADC_P			0xF7

#define BMP280_READ_DIG_PX			0x8E

#define	SLAVEADRESS					0x76

//Header with config value and adress