# History

2026/02/18 Change remark: Digits (=number of decimal places) are not documented, but working in WebIQ
2026/02/05 First version

# Introduction

When switching languages in an HMI, the text is displayed in the user's language. The same applies to switching units, where variable values ​​are converted into the user's units.

- HMI reads PLC   value: 	HMI converts from PLC unit to HMI unit

- HMI writes PLC  value: 	HMI converts from HMI unit to  PLC unit

Using unit classes in WebIQ is not very user-friendly.

- There is no editor to define the unit classes.

- Instead, a JSON file must be manually created.

- Unit classes are assigned using numbers, which makes assignment and readability very difficult. 
You must read the JSON file to find out the meaning of the class number.

| Without unit classes. “mm” clear unit string | Using unit classes. Encrypted class id |
| --- | --- |

- The Add-On: Unit Class Manager is an editor for configuring unit classes.

- The Add-On: Variable Manager provides input support for assigning unit classes.

| Input support for unit classes | Tooltip in table header show id and unit class name |
| --- | --- |

- A unit class must be created for each unit type (length, temperature, time, ...).

- Different unit classes can have different numbers of elements.

  - Temperature (K, °C, °F)

  - Distance (mm, inch)

  - Time (hour, minute, …)

- There are two possibilities to set a unit class

  - Set each unit class separately.
Use case: When you change the language to set all unit classes separately. This mean units in table rows are independent from each other. Example configuration:

| Unit ID | Temperature unit-class=1 | Length unit-class=2 |
| --- | --- | --- |
| 0 | K | mm |
| 1 | °C | inch |
| 2 | °F |  |

    - Set temperature = K : Set virtual:unitclass-1-adapter = 0

    - Set length = mm: Set virtual:unitclass-2-adapter = 0

Advantage: Class definitions are independent

Disadvantage: Multiple commands are necessary to set all units

  - Set all unit classes to the same number. 
Use case: All units of a language are listed in the same row. This means that the same unit may appear multiple times in the table because it is used in several languages.

| Unit ID | Temperature unit-class=1 | Length unit-class=2 |
| --- | --- | --- |
| 0 | K | mm |
| 1 | °C | inch |
| 2 | °F | inch |

    - UI-Action: UNITCLASS-ADAPTER: User specified Adapter Index = 1

(Temperature = °C, Length = inch)

Advantage: 	Only a single UI-action is necessary to set all units.

Disadvantage:	Units in table must be aligned

  - HINT: 
WebIQ has already predefined the unit names in the UI-Action xxx: 0=(metric), 1=(imperial). This pre-assignment is confusing because, although the names are predefined, there is no predefined definition for it. Furthermore, the user is completely free what he enters at index 0 and 1.

Therefore, my recommendation is: Always select “User specified” to avoid confusion if the predefined names do not match the users configuration.

| Predefined indices 0 and 1 | User specified index |
| --- | --- |

# Usage

|  | (1); Return to Add-On main menu. (2); Adds a new extra column with a unit class. (3); Load unit classes from WebIQ json file. (4); Load unit classes form browser local storage. (5); Save the unit class configuration (to browsers storage & clipboard). (6); Shows this help documentation. (7); Adds a new unit with Unit, Factor, Offset and Digits rows. (8); Deletes unit class with all units. (9); Deletes the clicked unit (all other units are not affected). |
| --- | --- |
| Screen shoot of GUI | Description of buttons |

The field digits is not documented, but used by WebIQ (v2.17.0).

HINT: To open the file unit-classes.json in notepad editor, 
run open-unit-class.bat from folder install-webiq-addon-v*

| Run open-unit-class.bat | Screen shoot of open-unit-class.bat |
| --- | --- |