# History

2026/02/05 First version

# Features

- Important variable attributes clearly (Interval, label, unit, min, max, ..) listed in one table

- Editing an attribute of multiple variables with one click, e.g. set communication interval for all selected variables.

- Cross reference list

- OpcUA variables check. o Is the variable still defined in the PLC? o PLC Data type changed?

# Usage

The Add-on uses the WebIQ Designer web server. That's why it works only if the project is opened in WebIQ Designer.

- Open your project in WebIQ Designer

- Install the Add-On

- Enter URL (e.g.: localhost:10124/addon) in your web browser
Hint: The port must be configured once in WebIQ Designer

- All changes in the Designer must be saved before they can be displayed in the Add-on.

# First login

At the first login, or when the login changes, you must provide a system user with password. Use 127.0.0.1 or localhost (which means local PC) instead of the IP address of your PC. 
HINT: User name and password are stored in the local storage your web browser.

# Selection of rows

- Before a command can be executed, the table rows to which it should be applied must be selected. The following options are available:

- Icons: Select all/Select none/Invert

- Select a range with the mouse:

  - Click first row, hold “Shift” key, Click last row

  - 2x Click first row, Click last row

  - Hold “Ctrl” key and click on a row, to toggle selection

# Sort columns

- Increasing and decreasing sort order

- Click on header row of the table to sort column

# Filter columns

- All filters are AND filters. This means that only rows containing all entered filter conditions are displayed.

- Filters are not case-sensitive.

- Multiple filter words in a single column are supported. A space (" ") serves as the field separator. Example:

| Column | Name |
| --- | --- |
| Filter string | AB EF => 2 words “AB” & “EF“ |
| Matching rows | ABcdEF |
|  | EFcdAB |
|  | aBEfcd |

# Cross Reference

- The cross-reference list is based on string comparison. Therefore, it can happen that too many references (e.g. a comment in a script) or not all occurrences (e.g. the item name is concatenated in a script (“stTest.” + “width”) of items are displayed.

- Although the cross-reference function counts all cross-references, it cannot list the path to certain cross-references because the program is not yet able to read all internal dependencies.

- This add-on analyzes WebIQ's undocumented internal storage format. Because this format can change, this add-on may no longer work with newer WebIQ versions.

There are 3 modes:

- Select multiple rows and click on button , which shows an info in the “check” column. 
E.g.  5x:MARTS => found 5 times “M”: Model “A”:Alarm “R”:Recipe “T”: Trend “S”:Script “-“:Not found
        3x:M-R-S   => found 3 times "M": Model "R":Recipe "S":Script

- Double click a row, which creates the cross-references for clicked item.

- Search for manually entered text.

  - Enable the checkbox (1)

  - Enter the text to search for (2)

  - Click button (3)

Use case: too many cross references found.

- An item name used as label.

- An item/item attribute in a composite widget is overwritten by a placeholder
Note: To avoid this, you can remove this item from the composite and leave it empty or 
use placeholder string, which does not exist as item name and makes it more readable for you 
e.g.  _Item_replaced_by_placeholder_  _Item_exported_as_attribute_

- An item name is used in a comment of a script

Use case: Variable not found.

- Composites

  - Placeholder only contains a part of the item name

    - Placeholder: "<%= item %>.Status_strUnitsVelocity"

  - Scripts

    - Item names are build with string operators

      - sVar = “myAxis” + “.Name”

      - sVar = “Axis.” + iCounter + “.Name”

- An item is used in a location that the add-on cannot decrypt because the WebIQ format is not documented.

# Set attribute.

Note: The update of changed attributes must be done manually by clicking “Update items” in “Process Data Manager” of WebIQ Designer.

- Select attribute (Label, Interval, …) from select box

- Hint: The column header of the selected attribute is highlighted

Set attribute

- Either type in your value in field with placeholder “Enter attribute value…” or

- Press button "..." to open the input assistance. All items starting with "@" (e.g. @AtStart) are reserved word. See table below the picture.

| Dialog attribute “Interval” | Dialog attribute “Name” |
| --- | --- |

Input dialog example for interval.

| Attribute |  | Keyword | Description |
| --- | --- | --- | --- |
| Label | @Identifier @1stWORD.1 @1stWORD.2 @1stWORD.-1 @1stWORD.-2 @REPLACE,search,replace @None |  | Use item name as label Use word 1 of item name from left Use word 1-2 of item name from left Use word 1 of item name from right Use word 1-2 of item name from right Replace search text by replace text Undefined |
| Interval | @None @AtStart |  | 100ms Read once at start |
| Unit | @None |  | Read units from json |
| Digits | @None |  | Undefined |
| Array Size |  |  | No reserved word exists |
| Min | @None |  | Undefined |
| Max | @None |  | Undefined |

# Check OpcUA items

This feature checks the following points:

- Does the referenced PLC variable still exist?

- Does the WebIQ item have a different type than the PLC variable?

- PLC Data type is LINT or LWORD When the integer in PLC has more than 15 digits, the value on HMI is not correct.

  - In WebIQ JavaScript is used to store variable values

  - JavaScript only knows data type NUMBER, which is a real number with 15 significant digits o It is used to store real and integer numbers

  - Integer values with more than 15 digits are rounded identically to real numbers Example: PLC: 9223372036854775123 => HMI: 9223372036854775000

The result of the check is displayed in table column "Check"

For this check to be performed, the following is required:

- the correct PLC project must be running on the ctrlX

- the PC must be communicating with the ctrlX

Screenshot with check results

| Error description | Error string |
| --- | --- |
| Plc variable not found Remove variable from HMI or add it to PLC. | no PLC variable |
| Plc data type changed Ensure that data type on HMI and PLC is the same. Either change the type in HMI or PLC. | Plc type: e.g. Plc type: float |
| Range of PLC variable is bigger than HMI variable Ensure, that you do not use integers which have more than 15 digits | Plc range: e.g. Plc range: i64 |