Check of regulation loops using frequency response analysis with ctrlX DRIVE

In the following we show to you how you may check the regulation loops using the frequency response analysis with ctrlX DRIVE via the ctrlX DRIVE Engineering tool.

Prerequisites

A connection to the ctrlX DRIVE has been successfully established, the device is successfully powered on, the first setup routine, the initial movements and the procedure for autotuning the regulation loops have been executed. As well the engineering tool ctrlX DRIVE Engineering has been started. In the following sequence the procedure for checking the regulation loops using frequency response analysis is shown step by step.

Press the button "Start easy startup mode".

Fig. : Start “Easy Startup Mode”

Next select menu: Frequency response analysis.

Fig. : Select “Frequency response analysis”

1. Check proper settings of filters

Use analysis of "Speed-controlled system”.

1st select the "Speed-controlled system” as type of frequency response measurement, 2nd press this button (2) and 3rd take over the suggested excitation amplitude by pressing this button (3). Then 4th enable the drive by 5th acknowledging the “OK” button.

Fig. : Select analysis of "Speed-controlled system”

Then 6th start the measurement.

Fig. : Start measurement

7th display the result.

Fig. : Display result of frequency response measurement

Then check the result if one or more dominant resonance frequencies can be seen. In here at 2000 Hz a dominant resonance frequency is present.

Fig. : Check result of frequency response measurement

Check if dominant resonance frequency or frequencies are properly suppressed.

Fig. : Filter settings  resonance frequency at 2000 Hz is suppressed

2. Check proper settings of speed loop

1st select "Analysis of speed control loop” as type of frequency response measurement.

Fig. : Select “Analysis of speed control loop”

Then 2nd press this button (2) and 3rd take over the suggested excitation amplitude by pressing this button (3). Then 4th enable the drive by 5th acknowledging the “OK” button.

Fig. : Sequence for starting “Analysis of speed control loop”

Press button "Start measurement"!

Fig. : Start measurement

7th display the result.

Fig. : Display result

8th scale the result scope diagrams automatically!

Fig. : Result of “Analysis of speed control loop”

9th scroll bar that blue signals can be seen. 10th double click on one of the blue signals. This will cause the -3dB frequency to be shown. If 11th the box is checked the speed loop bandwidth will be indicated automatically.

Fig. : Sequence for checking the result of “Analysis of speed control loop”

12th press button (12) to show cursors. 13th move cursor 1 to position where purple line meets “0” line in amplitude. 14th move cursor 2 to position where purple line meets “-180” line in phase.

Fig. : Evaluate “Analysis of speed control loop”

15th press “Show cursor positions”!

Fig. : Show cursor positions

Fig. : Evaluated “Analysis of speed control loop”

The green box shows the bandwidth of the closed speed loop and gives an indication of the behavior of the speed loop up to which frequency being able to regulate properly to command, disturbance and load changes.

The red box shows the stability of the regulation loop settings. The so called phase margin is calculated by

180° + (phase value shown at cursor 1) = (in here) 180° + (-118,5°) = 61,5°

For good behavior this value should be in between

  - 40° to 70° for good command response behavior

  - 20° to 50° for good load response behavior

  - 40° to 60° for well balanced loop settings

The blue box shows the stability of the regulation loop settings. The so called amplitude margin is calculated by

- (amplitude value shown at cursor 2) = (in here) - (- 9,8 dB) = 9,8 dB

For good behavior this value should be

  - at minimum 5 dB

  - usually at least 10 dB for well balanced loop settings

After the frequency, marked for amplitude margin, the blue line should not exceed the -10 dB line at higher frequencies again except on the limits of the diagram at 4000 Hz.

So the settings here are quite well and close to the optimum.

3. Check settings of position loop

1st select the "Position ctrl loop” as type of frequency response measurement, 2nd press this button (2) and 3rd take over the suggested excitation amplitude by pressing this button (3). Then 4th enable the drive by 5th acknowledging the “OK” button. Then 6th “Start measurement”!

Fig. : Sequence for frequency response analysis of position loop (1)

7th display the result!

Fig. : Sequence for frequency response analysis of position loop (2)

Fig. : Result of position loop frequency response analysis

The – 3dB frequency line shows the bandwidth of the position loop and gives an indication of the behavior of the position loop up to which frequency being able to regulate properly to command, disturbance and load changes.

4. Check settings of current loop

Usually only for kit motors and 3rd party motors the current loop settings need to be checked. In order to do so 1st select the "Current ctrl loop” as type of frequency response measurement, 2nd press this button (2) and 3rd take over the suggested excitation amplitude by pressing this button (3). Then 4th enable the drive by 5th acknowledging the “OK” button. Then 6th “Start measurement”! And 7th “Display result”.

Fig. : Sequence for frequency response analysis of current loop (1)

Fig. : Sequence for frequency response analysis of current loop (2)

Fig. : Result of frequency response analysis of current loop

The – 3dB frequency line shows the bandwidth of the current loop and gives an indication of the behavior of the current loop up to which frequency being able to regulate properly. Usually this figure should be higher than 1000 Hz.