Below are the steps to install OPC DA Tunneler:

- Copy all Tunneler files to station machines. Below is the list of files:

- RegisterDependencies.bat   Batch file which will install all dependency.

- Rbei.WatchDog.exe  Watch dog app to monitor Rbei.OpcDA.exe

- Rbei.OpcDA.exe  OPC DA Tunneler

- CONFIG.INI  Configuration file

- MSWINSCK.OCX   For socket communication

- OPCDAAuto.dll   For OPC DA connection

- capicom.dll   For Encryption handling

- Open the CONFIG.INI file and configure the parameters i.e. OPC DA Server name and port no. for communication with device Bridge application. Other parameters can be kept as default configured values.

- Save and close the file

- Run the RegisterDependencies.bat . This will register all dependency and register Rbei.WatchDog.exe to auto start on system restart. This watchdog will take care of starting Rbei.OpcDA.exe

Note: For Win NT OS, please register  Rbei.WatchDog.exe to below path

HKEY_LOCAL_MACHINE\SOFTWARE\Microsoft\Windows\CurrentVersion\Run