Handling template for

ctrlX AUTOMATION

Titel

Basic-Hints-Template-Handling

Documentation type

User description

Docu-Type

Internal filing note

Application description

Purpose of the documentation?

Table of contents

1.	Change history	5

V2.6.4 October 2024	5

V2.6.3 August 2024	5

V1.20.1 April 2024	6

V1.20.0 December 2023	6

V1.0 June 21, 2022	6

V0.4 April 14, 2022	7

V0.3 March 18, 2022	7

2.	Introduction	8

2.1	General hints	8

3.	Requirements	9

3.1	Apps	9

3.2	Libraries	9

4.	First Startup	10

4.1	ctrlX CORE / ctrlX OS	10

4.2	PLC Engineering	10

4.3	WebIQ	11

4.3.1	Load WebIQ project	11

4.3.2	Edit WebIQ project	12

5.	Known bugs	14

5.1	Motion App	14

5.2	HMI	14

5.3	PLC	14

5.4	Restrictions	14

6.	Features	15

7.	Features in detail	16

7.1	Choose method for power on via fieldbus	16

7.2	Adapt axis in ref signal	16

7.3	Adapt axis & kinematic configuration	17

7.4	Adapt axis and kinematic settings	18

Axis settings	18

Kinematic settings	19

7.5	Configure kinematic switches	20

7.6	Configure common features	20

7.7	PLC state machine	21

7.8	Code generation for inputs/outputs	22

7.9	Display active step of PLC automatic program in HMI	24

7.10	Virtual keypad	26

7.10.1	Configuration of alphanumerical keypad	27

7.11	Help documentation	28

7.12	Charts	28

7.13	Oscilloscope	28

7.14	Teach point table configuration	29

7.15	Alarm/Warning/Info Management	30

7.16	Debug> Plc/Python	31

7.17	Debug> Debug Funcs	32

7.18	Group rights	32

Features	33

Implementation details	34

7.19	Check OpcUa items	34

8.	WebIQ	35

8.1	Documentation	35

8.2	Setup development environment (reverse proxy)	35

8.3	Package Manager	36

8.4	Color Themes	36

8.4.1	Color schema 1 Corporate Design Rexroth	36

8.4.2	Color schema 2 Dark Theme	37

8.5	Composites	38

# Change history

### V2.6.4 Februar 2025

- PLC:

  - Add unit conversion in FB_TableMgr()

  - Bugfix: mCheckTeachPoints() check only valid group indices

  - Bugfixes table: immediate updating of the tab buttons when changing op modes

  - Bugfix: Switching off “safe areas” was not possible

  - Bugfix: Disable/Enable watchdog was not working correctly

  - Bugfix: Support long kinematic names

  - Add heartbeat feature for jogging to detect communication interrupt

- HMI:

  - Dialog boxes and virtual keypad can now be moved and resized

  - Enhanced input features for virtual keypad (see chapter 7.10 Virtual keypad)

  - Bugfix: enable “Debug>Check Ua items” for published project

  - Workaround: set plc runtime-settings automatically because they are sometimes lost (bug) after reboot of ctrlX

  - Add default settings for group rights

  - Validate group rights settings when saving

  - New page: Debug> Config HMI

  - Changed to new Material design widgets, while a later change is not supported

  - The new WebIQ Theme Manager is not yet used, because of missing documentation of the Theme Manager.

### V2.6.3 August 2024

- PLC:

  - Disable watchdog in INIT mode and for ctrlX COREvirtual
When booting many apps are started, which results in a high load and long

PLC cycle times. Therefore, the watchdog is disabled in MODE_INIT.

ctrlX COREvirtual has no real time OS => watchdog must always be disabled.

  - Added support of non cartesian kinematics

- HMI:

  - Incompatible OpcUA server changes in ctrlX PLC app 2.6

    - Set variables on data layer path: plc/admin/runtime-settings/symbol-settings

{ "enumRepresentation": "BASETYPE", "arrayDisplayName": "OFF" }

    - The HMI displays a warning when the values of the variables are wrong.

  - Configuration help pages changed. See 7.11 Help documentation

  - WebIQ auto login user: changed to “operator”

  - Auto login for 3D viewer and ctrlX OS diagnostic page

Administrator features

  - Added editors in WebIQ for

    - Axis limits & settings (max. jog velocity, enable jog & referencing, …)
see 7.4 Adapt axis and kinematic settings

    - Kinematic limits & settings (min. max. position, enable jog, …)
see 7.4 Adapt axis and kinematic settings

- PLC & HMI config settings
see 7.6 Configure common features

- Group rights (define who can use a feature of the HMI program)

see 7.18 Group rights

  - Check if OpcUa items exists or data type changed
see 7.19 Check OpcUa items

### V1.20.1 April 2024

- PLC & HMI:

  - Update table functionality to enable simultaneous editing with multiple web clients

  - Add 2nd recipe which can be edited while machine is running

  - Further separation of Base and Customer code/variables

  - Bugfix to enable jogging of free kinematic axes

This means incompatible changes in HMI and PLC code. To get your existing code of a previous template running again, import the code from your existing project into the new HMI & PLC template. In PLC add “stRecipe.” to the following variables:

| Old variable | New variable |
| --- | --- |
| arSafeZone | stRecipe.arSafeZone |
| arTeachPoints | stRecipe.arTeachPoints |
| stSetUp | stRecipe.stSetUp |
| strScriptName | stRecipe.strScriptName |

- HMI:

  - Updates of WebIQ packages

  - Package cxVisuals:

    - add widget cx-select

    - cx-table: implement feature to display WebIQ array

    - cx-jog: bugfix to enable jogging of free kinematic axes

### V1.20.0 December 2023

- PLC:

  - Separate Base and Customer code/variables

  - Use original CXA_MOTIONINTERFACE library, to avoid additional effort on updating

  - Optimize source code, Restructure, and rename some variables

  - GVL_CFG as central place to enable/configure features

- HMI:

  - Add Color Theme manager

  - IDE Visual Coding via iframe

  - System information: Axes, kinematics, Apps, Licenses & Host info

  - Replace composite widgets by package widgets

  - Add charts and oscilloscope feature (controlled by PLC code)

  - Display active step of PLC automatic program in HMI (controlled by PLC code)

  - config.js as central place to enable/configure features

### V1.0 June 21, 2022

- PLC:

  - Implement State machine in ST, this allows customer specific code extensions

  - Workaround for Start/Pause in mode Automatic

  - Code optimization for stability and better maintainability

- HMI:

  - Revision of GUI design

### V0.4 April 14, 2022

- PLC:

  - Improvements in state machine

  - Code optimization for stability and better maintainability

  - Update of internal visualization

  - Additional checks in code generator for Input/Output force mechanism

  - There is still a bug in KinInterface library, that some errors could not be deleted.

### V0.3 March 18, 2022

- PLC:

  - Bugfix in state machine. Now all errors can be cleared

  - Code optimization for stability and better maintainability

  - Step mode in PLC automatic program for debugging

  - Configuration of motion data like max. Jog velocity, Kinematic limits

  - Usage of Safe Areas in ctrlX MOTION

  - Code generator for Input/Outputs force mechanism

  - EuroMap variables are moved to variables ix: ST_INPUT and qx: ST_OUTPUT

- HMI:

  - Revision of GUI design

  - Update of changed OPC UA variable names

  - Display hint, when WebIQ screen is no editable, because of wrong op mode or user.

# Introduction

The following documentation describes how to use the HMI-Template (for Handling). It helps to setup the template on your ctrlX OS, ctrlX CORE or ctrlX COREvirtual. Furthermore, it explains how the template is created and gives tips how to modify or create your own HMI Solution. Start at this point if you want to install the template for your first time.

## General hints

This template is based on an example of a customer’s project. Therefore, keep in mind the following things:

- Example code uses specific axes, kinematic names.

- FB_Machine_Base is protected, to prevent any changes of the code.

The template is created with ctrlX OS version 2.6.3. There is no guarantee that the code will compile successfully in newer ctrlX OS versions without adaptation.

To understand this documentation, you must be familiar with the basics of ctrlX AUTOMATION and WebIQ.

See How-To section in ctrlX AUTOAMATION Community: https://developer.community.boschrexroth.com/t5/Store-and-How-to/Collection-of-how-to-videos-blogs-and-examples-for-ctrlX/ba-p/12343

This entry is a collection of very helpful videos and documentation for wide range of know how.

See PLC Engineering manual

https://docs.automation.boschrexroth.com/doc/154626822/ctrlx-plc-engineering-system/latest/en/

See WebIQ manual
https://www.smart-hmi.de/user/download/deliver/docs/documentation-manual-webiq-designer-2.13-e58c/index.html

ctrlX AUTOMATION - IDE app Textual and Visual programming

Episode #10 PYTHON Programming

Graphical and Textual Python Programming using IDE App [DOCU] - [VIDEO]

## Update from a previous template version

In new versions, optimizations and new features are added to the template. To be able to use these new features, the following procedure is strongly recommended for HMI and PLC:

- Export your specific program code from the previous template version.

- Open the current template version.

- Import your specific program code.

- Check your imported program code for necessary changes.

HINT:

We strongly advise against going the other way round by incorporating the changes to the template into the customer project. Practical experience shows that the recommended steps are the more effective and faster way with fewer sources of error.

# Requirements

## Apps

The following apps are necessary to run all features of the provided example. You need the licenses for all apps. For WebIQ Server you need a license with 500 items or more.

Mandatory

- EtherCAT Master (Get access to I/O’s & drives via EtherCAT fieldbus)

- Motion (Set up, commission, and move axes and kinematics)

- OPC UA server (communication to WebIQ)

- PLC (develop and run PLC applications)

- WebIQ Server App (HMI server 2.15.10 or newer) 
(To edit HMI project you need to install WebIQ designer on your PC)

Optional

- 3D Viewer (Display 3D model of moving kinematics)

- IDE (Integrated develop environment for python with file manager)

- Python Runtime (to execute python scripts)

## Libraries

These following 2 libraries are included in the project archive of this Template. The provide powerful function, but they are not part of the ctrlX PLC Engineering installation:

- App_CommonData

- App_Tools

# First Startup

## ctrlX CORE / ctrlX OS

- Install apps and licenses (see chapter 3.1 Apps)

- To keep template simple, it uses the default user and password boschrexroth

- In a real project, for security reasons, you should create a user only with OpcUA rights for OpcUA communication.

- The same applies for the user used for the REST API communication.

To load axis, kinematics definition and python scripts:

- Switch system state to “Setup” (1)

- Click “Manage App Data” (2)

- Click “…” -> “Upload” (3) and select AppDataTemplate-10Ax2Kin. The archive contains 10 axes and 2 kinematics.

- Open branch Motion to view the imported axes and kinematics.

Figure 1: Manage App Data

## PLC Engineering

When you are not familiar with PLC Engineering, see chapter 2.1 General hints

- Unzip Base-Template-Handling-V*.zip and open PLC project archive with ctrlX PLC Engineering.

- Open project: File> Open Project…

- Check if project device matches your device (virtual or real ctrlX CORE)

  - Right click ctrlX CORE… in the project tree

  - Select Update device

  - Select device you are using

  - Click on button Update device

  - Click on button Close

- Restriction for ctrlX COREvirtual: Only WebIQ Runtime is supported

- Check IP settings:

  - Right click ctrlX CORE… in the project tree

  - Select Communication settings…

  - Enter IP or select it from selection box

  - Click on button Apply to verify connection

- Login to ctrlX CORE

  - Online> Login (Provide user & password)

- Start PLC program

  - Debug> Start

Hint:

- Dependent on your program requirements, you might have to adapt Task Configuration.

- ctrlX COREvirtual has no real time operating system, therefore the PLC program automatically disables the watchdog.

- Only necessary with real axes:

- Currently the power activation is hardware dependent

- Check which option mPower1()-mPower4() meets your setup

- Set PersistentVars.iPowerOpt to 1-4 to select a power activation method

- When no option matches, create your own mPower5()

So far, no performance tests and analyses have been carried out. Therefore, no experience about shorter and longer cycle times is available.

## WebIQ

When you are not familiar with WebIQ see chapter 2.1 General hints

## Load WebIQ project

Login to ctrlX OS web UI and click on “WebIQ Runtime Manager”. The first time after installation of WebIQ on ctrlX OS you must define a user and password for login (e.g., User: admin password: boschrexroth1). This user is valid for “Runtime Manager” and “Designer”.

- In menu item Hmi Projects (1) press Upload (2) button and select the WebIQ project

- Press … and select Start (3) to run the project

- Press … and select Open (4) to display the project in browser

Hint:	When you forget the password, you must delete and reinstall the WebIQ app. This means all WebIQ projects on ctrlX OS are deleted.

Hint:	If your WebIQ version is newer than that of the WebIQ PRoject, you must update the project. See 4.3.2 Edit WebIQ project

Figure 2: WebIQ Runtime Manager

## Edit WebIQ project

The WebIQ Designer is WebIQ's development tool. It is only needed when project needs to be edited. If you have installed a newer version of WebIQ, you must update the project by loading, updating, and saving it.

To connect the WebIQ Designer to the ctrlX OS, you must add some parameters. Create a new shortcut for the WebIQ Designer application and click on Properties. In this example the used IP address is “10.52.244.3“. At download page of WebIQ in ctrlX Store, there is a documentation which describes this in detail. There is even a script, doing the work for you.

You must add:

“<WebIQ_Installation_path>\Designer\webiq-designer.exe" --ws-host=10.52.244.3 --wsport=10123 to the target input field and apply the changes.

Figure 3: Shortcut Properties

Start WebIQ Designer with the shortcut and login.

- The first time after installation of WebIQ on ctrlX OS you must define a user and password for login. Th user is used by “Runtime Manager” and “Designer”.

- Import Project HMI-Template*.zip

- Load Project

- Install packages (containing additional features) via WebIQ Package Manager.

- Publish Project

- Start program (see 4.3.1 Load WebIQ project)

To establish the OPC UA communication the OPC UA client certificate of SmartHMI must be trusted in ctrlX OS. This must be done only during the first startup.

To trust the certificate, open ctrlX OS Web UI, go into Settings > Certificates & Keys > OPC UA Server. Click on “…” of the Smart HMI certificate and select “Trust”.

Figure 4: Trusted OPC UA Certificates

Start the machine and test the different functionalities of the Template. For example, you could modify the Teachpoints. Keep in mind that the most inputs are only possible in mode manual or setup. To start the movement, switch to automatic mode and press the Start button.

# Known bugs

## Motion App

- For further information see release notes of ctrlX MOTION app https://docs.automation.boschrexroth.com/iirds/cdp-metadata.boschrexroth.de~iiDC~Product-ctrlX-Motion-App/.

## HMI

- Unicode characters (e.g., special Asian chars) can be exchanged via OPC UA, but only for PLC variables of type WSTRING.

- Some custom widgets are not yet using the theme manager colors.

## PLC

- Sometime errors can only be cleared by resetting the PLC program.

- Sometime a kinematic configuration is not loaded correctly. PLC needs to be reset warm.

- Very rarely, activating an OP fails and must be selected a second time.

- Very rarely, override value is not used correct (slow movement). Change override to fix.

- The PLC internal visualization is not up to date; therefore, some screens/functions are not working.

## Restrictions

- In mode Automatic and Manual, the Safe Areas are used if switched on.

- In mode Setup, the Safe Areas are not used. Even if switched on.

- FB_Machine_Base contains the state machine and some basic features helpful for many machines. This FB is password protected, to ensure that nobody can change the FB. While you can overwrite every method in the extended FB, it is your decision if you use the features of the base FB or you overwrite it with the features you need in your project.

- Drive based homing not fully implemented yet.

# Features

- Axis/Kin configuration checking at startup of PLC program. Problems are shown in HMI.

- Automatic load DC bus and switch drives to Ab

- Last active recipe is automatically loaded at ctrlX OS startup

- Jog widget for single axes and kinematic (HMI menu item: Commissioning> Jog)

- Display and force digital IO’s via HMI (Commissioning> Inputs/Outputs)

- Template can be used without a kinematic (only with axes), too.

- Table for editing teach points (Recipe/Teachpoints)

- Predefined HMI languages: German, English and Turkish (Settings>General)

- Communication check: HMI is locked if not connected to PLC
(reasons: 1. security 2. some features are only working when PLC is running)

- Alarm handling: Active, history and ctrlX OS diagnostics. (Diagnostics)

- HMI User Manager which allows to add, edit, delete… users (Settings>User Manager)

- Responsive design (Min. requested resolution tablet)

- PLC example code moving between 3 different points 
(Pos-Home, Pos-Mold-Up, Pos-User-10)

See FB_Machine_Customer.mAutoPlc()

- PLC example code to manage Python interpreter (create instance, start script, …)
See FB_Machine_Customer.mAutoPython()

- Recipe handling in PLC with Open, Save, SaveAs and Delete in mode Manual & Setup
(Recipe) (PLC recipe structure stRecipe)

- 3D viewer showing the moving kinematics

- IDE Coding in iframe

- Oscilloscope and chart graphics

- Editors for Group Rights, Axis & Kinematic feature & limit configuration

# Features in detail

The following describes how to use some specially implemented features in more detail.

There is a powerful feature in PLC Engineering, how you can find any item in the PLC project tree, by the following steps:

- Click anywhere in the project tree, to active the window

- As soon, as you type the first character, a window with matching elements is opening

## Choose method for power on via fieldbus

When real ctrlX DRIVEs are connected to the fieldbus the DC bus can be loaded and drives status changed from bb to Ab via the fieldbus. Therefore, example programs in the mCyclic_Active of the FB_Machine_Customer can be chosen via variable iPowerOpt. Each is representing a special hardware topology.

- mPower1
Power supply must be a single axis XCS and two more slaves must be attached

- mPower2
Power supply must be a double axis XCD and one more slave must be attached

- mPower3
Power supply must be a single axis XCS and 6 more slaves must be attached

- mPower4
Power supply must be a single axis XCS and 3 more slaves must be attached

## Adapt axis in ref signal

In all mPower methods the checking if axes are in ref is implemented. For virtual axes it will automatically be set to true. As for real axes the information is missing for now in the PLC, customer will have to extend the cyclic data of all axes with parameter S-0-0403 and use each Bit 0 in the code.

//Check if each single axis is in reference
arAxExt_gb[AxisX.AxisNo].InRef := FALSE OR_ELSE NOT bRealAxis; // Exchange FALSE to S-0-0403 Bit0 of X_Axis

## Adapt axis & kinematic configuration

To get familiar with the template, you should use the default axis settings. But later you might have to add axes or change their names.

Hint: When you change axis names, you must adapt them in 3D viewer model as well.

Hint: You can also add individual axes to an empty PLC kinematic structure. 
         This means that individual axes can also be jogged and referenced via WebIQ jog widget
         For more info see comment in GlobalKinematicsDefines

The Init procedure of the PLC program verifies if the Motion configuration matches the PLC. If a configuration error is detected, the HMI shows an information. To run the program, you must fix the error first. For example, you have defined an axis name that does not exist.

Figure 5: Config error

When a config error happens, check the following:

| Axis configuration | Kinematic configuration |
| --- | --- |
| 1. AxisName must match the name defined in motion configuration. 2. AxisNo must be unique in range defined in MOTIF_CONFIG. 3. All existing axes must be defined here | 1. KinName must match the name defined in motion configuration. 2. GroupNo must be unique in range defined in MOTIF_CONFIG. 3. Axes assigned to kinematic must exist |

After fixing the problem, do a complete download of the PLC program and reload the HMI in the browser.

| Config Error: | TE_AxisInterfaceMainProg |
| --- | --- |
| Definition: | MotionInterface/cfg_axes-kinematic/GlobalAxisDefines/ |

Figure 6: Config axes

| Config Error: | TE_KinInterfaceMainProg |
| --- | --- |
| Definition: | MotionInterface/cfg_axes-kinematic/GlobalKinematicsDefines/ |

Figure 7: Config kinematics

## Adapt axis and kinematic settings

For the commissioning of a real machine, some important information is still missing in the data layer of the ctrlX. As a workaround, this data is saved in the PLC and can be edited in the WebIQ HMI. See Commissioning> Axis and Commissioning> Kinematics.

When the PLC program is started, the saved setting is compared with the existing axes and kinematics to determine changes. If a deviation is detected, an administrator must adapt the configuration.

### Axis settings

| Label | Meaning |
| --- | --- |
| Axis PROJECT | Name of all axes defined in project |
| Axis CFG | Name of all axes stored in config file |
|  | Displaying both axis name enables you to detect differences between project and stored configuration. If the configuration does not match the project: - Revise settings - Copy axis names with button "Copy name" - Save changes |
| Jog. Rel. | Enables relative Jog in widget cx-jog |
| Ref. Abs. | Enables setting of absolute reference in widget cx-jog |
| Ref. Drive | Enables drive-based referencing in widget cx-jog |
| Ref. Drive Pos. | Show position input field for drive-based referencing. While all parameters for drive-based homing are stored in the drive, sending the position to the drive must be implemented in FB_Machine_Customer.mSetDriveHomingParams() |
| Mode in Auto | OP mode, which is set, when axis does not belong to a kinematic |
| Kin. axis? | Checked if axis is part of any kinematic |

### Kinematic settings

| Label | Meaning |
| --- | --- |
| KIN_X Min. Pos [] | Min. position of kinematic axis X |
| KIN_Y Min. Pos [] | Min. position of kinematic axis Y |
| KIN_Z Min. Pos [] | Min. position of kinematic axis Z |
| * Max. Pos [] | Max. position of other kinematic axes |
| Jog. Rel. | Enables relative Jog in widget cx-jog |
| Max. Jog Vel. [] | Max. jog velocity for kinematic |

To set other application specific settings, see FB_Machine_Customer/00_Init/mUserCfg.

## Configure kinematic switches

In FB_Machine_Customer/00_Init/mUserCfg switches can be configured. These switches are displayed in the jog widget of the HMI, e.g., to activate compressed air or gripper in OP mode Manual. To display a switch set a reference to a BOOL variable and define the label for the HMI button.

## Configure common features

In WebIQ runtime open “Debug> Debug Funcs”. Here you can set PLC features.

PLC: See folder PersistentVars in Plc engineering to configure PLC features. Here some examples:

| Flag | Description |
| --- | --- |
| bDisplayAxesPos | Display axes position instead of kinematic position, while kinematic position is invalid, when not grouped |
| bTeachPointEditInAutoMode | Allow/Disable editing teachpoints in OP mode automatic |
| bHoldJogButton | jog button must be hold until distance reached |

WebIQ: Open local script cfg in Code Manger of WebIQ Designer. Here some examples:

| Flag | Description |
| --- | --- |
| sCfgVirtKeypadStart | Enable virtual keypad |
| iCfgHeartbeatInterval | Heartbeat interval |
| iCfgStartupDelay | Startup delay for HMI |
| URL_3D_VIEWER | Url to display 3D viewer |
| URL_TIGER | Url to display IDE |
| arIdeUserGroupsShow cssHideIdeCtrls | Currently there is no feature to hide specific controls in IDE app. You can use these two variables to configure which controls are hidden for certain user groups. |

## PLC state machine

At the beginning the GAT compact status machine was copied 1:1 from the MLC. Below you see the logic of the state machine.

Figure 8: PLC state machine

Since the original graphical implementation could not separate basis from user code, we changed to Structured Text. This results in the following advantages:

- Separation of base and user code. => Easy version update

- Entry and exit methods can be active for several PLC cycles instead of only 1 cycle before.

Although the inheritance of object orientation is used here, the required knowledge to use it, is very basic. All necessary information’s are included as a comment in the source code. All entry, active and exit methods have the same structure.

Figure 9: PLC example mManual_1_Active

## Code generation for inputs/outputs

The template includes a mechanism that allows forcing of inputs and outputs via the HMI. The code required for this feature is generated by a Python script in PLC Engineering. No additional work, like adding OPC UA variables or HMI screens, is required. So, you can save a lot of implementation time.

By default, digital IO is automatically detected, and the code is generated. Alternatively, you can still manually export the digital IO configuration.

You must accomplish these 5 steps to use this mechanism:

Step 1: Copy python code into a file e.g., makeCode_ForceIo.py (only once)

Step 2: Definition of the variables for the digital inputs/outputs

Step 3: Save PLC project

Step 4: Only manual export: Export every single digital IO into a separate csv file

Step 5: Start the Python script

Step 6: Adapt bDeadmenSwitch in MachinePrg (only once)

Step 1: Copy python code into a file

Creating the script file must be done only once. Afterwards the script can be used again.

- Open the POU makeCode_ForceIo in PLC engineering

- Copy complete python code into a text file (e.g., makeCode_ForceIo.py)

- Optional:

  - Set variable iMapAllIOs. 
0=do not map IOs without name 
1=map all IOs even the unnamed IOs. When no IO name is defined the bit address is used as IO name. e.g., IX50_0

Manual export of CSV files:

- To enable manual export in script: Set variable iAutoDetectIO=0

- Set the variable strCsvDir to the directory where you saved the CSV files. Therefor no dialog asks you to select the files.

Figure 10: PLC engineering Python script

Step 2: Definition of the variables for the digital inputs/outputs

Open the configuration mapping page. Enter the variable name of inputs/outputs into the description field of the input/output page. Keep in mind to create valid PLC variable names. If you add a * to the name (e.g., EM_Reject*), it is not exported to the symbol file.

The column Variable must not be filled in. This prevents writing to the IOs via 2 different paths.

Figure 11: Names of I/O

Step 3: Save PLC project

The changed digital IO variable names changes are only used, when the project is saved, before the python script is executed.

Step 4: Only manual export: Export every single digital IO into a separate csv file

This step is not necessary in default mode of script, where the digital IOs are automatically recognized.

Right click digital IO, select Export mappings to CSV… and provide a valid file name.

| Figure 12: Digital IO nodes | Hint: All csv files must be in the same folder Do not export any other module than digital IO’s. |
| --- | --- |

Step 5: Start the Python script

In PLC Engineering

- Click on Tools/Scripting/Execute Script File

Figure 13: Execute script file

- Start the python script, by selecting it within the file browser

- Manual export only: 
If you haven’t set strCsvDir, select any of the exported csv files and confirm with Open

- In the message window, check whether the code generation was successful and which DUTs, GVLs and POUs were updated.

- Compile the program. When compile errors are thrown, fix the invalid variable names. Start creation process again.

Step 6: Adapt bDeadmenSwitch in MachinePrg

Since not every machine uses a dead men switch or the signal is taken from a any source, you must define the source of the signal in MachinePrg().

// activate, when there is NO deadman input //bDeadmenSwitch := TRUE; // activite, when there is a deadman digital input bDeadmenSwitch := ix.Deadmen_Switch;

## Display active step of PLC automatic program in HMI

The template includes a mechanism to display the program steps in a table. The current step is highlighted. The displayed text is extracted from the ST program code.

Figure 14: Display program steps

You must accomplish these 5 steps to use this mechanism:

Step 1: Copy python code into a file e.g., makeCode_PrgSteps.py (only once)

Step 2: Define a text for every main step of state machine

Step 3: Start the Python script

Step 4: Login with Download

Step 1: Copy python code into a file

Creating the script file must be done only once. Afterwards the script can be used again.

- Open the POU makeCode_PrgSteps in PLC engineering

- Copy complete python code and into a text file (e.g., makeCode_PrgSteps.py)

Step 2: Define the text for every main step of state machine

- Open the POU FB_Machine_Customer.mAutoPlc

- Add you comment starting with //PRG in the same line as the CASE value of iAutoStep

Figure 15: Edit program steps

Step 3: Start the Python script

See chapter 7.8 Code generation for inputs/outputs Step 4: Start the Python script

The script generates some variables in GVL_PrgCode

Step 4: Login with Download

“Login with Online change” is not sufficient to update the information on HMI.

## Virtual keypad

The template uses a virtual keypad from WebIQ package cx-tools. Only the widget cx-input, uses this keypad. It has the following advantages:

- It can be enabled/disabled at runtime (Settings> General Settings)

- Min./Max. Limits can use variables which are calculated in PLC at runtime

- Max. input length for text

- Numpad or alphanumeric keypad automatically selected, dependent on item type

- Easy configuration of additional languages

- Enter upper chars by swiping up high distance (or Shift key)

- Enter lower chars by tap or swiping up low distance

- Keys with multiple characters are marked

  - Display additional characters by swiping to right

| Image | Description |
| --- | --- |
|  | - Bar at bottom displayed when additional characters are available - Display additional characters by swiping to right |
|  | - Enter lower character by tap or swiping up a low distance - Enter upper character by swiping up high distance |

## Configuration of alphanumerical keypad

Features of alphanumerical keypad:

- Letter page (char)

  - Each row contains lower and upper

  - Do not define more than 5 rows, to avoid small keys

  - Characters are separated by at least one space

- Number special characters page (number)

  - Same setup as char

  - Can be empty when not used/displayed

- Normal and password input

- Function keys

| #cancel cancel key #ok ok key #¦< move cursor to start #>¦ move cursor to end #<< move cursor to previous word #>> move cursor to next word #< move cursor to previous char #> move cursor to next char | #abc display keypad character page #123 display keypad number page #shift shift key #<del delete key #clear clear complete input #space space key #+- +/- key |
| --- | --- |

- Predefined languages (en/de) can be overwritten by adding cxt.keypadAddLanguage("de", myDE) in local script start-up

The keys of the virtual keypad are defined within a json object.

- Separate key characters by at least one space

- To add multiple characters to one key (lower or upper) combine the characters by a tabulator character. E.g., “a\tä\tÄ b c”

myDE: { char: [ { lower: "° 1 2 3 4 5 6 7 8 9 0 ß", upper: '^ ! " § $ % & / ( ) = ?' }, // you can add more array items (=keypad rows) { lower: "#shift <\t¦ y x c v b n m , . -", upper: "#shift >\t¦ Y X C V B N M ; : _" } ], number: [ // same format as section char ], header: { lower: "#¦< #<< #< #clear #> #>> #>¦" }, footer: { lower: "#cancel #abc #space #123 #ok" } },

## Help documentation

The template support displaying help information in multiple languages (Click “?” in header of template). If help of active language is not available, the help of default language is displayed, if available. (Button can be hidden see chapter 7.6 Configure common features)

The following steps are necessary to connect a help file to a screen:

- Insert local script showHelpFile as 1st child on the screen to which the file belongs

- Name of screen control must be used as file name (without extension .htm & .pdf)

- Create the help file (e.g. chart.htm or chart.pdf)

- Copy the file into the language folders

Folder: %appdata%\webiq-designer\workspace\docs\help\<LanguageFolder>

- When both files exist, the pdf file is displayed.

The <LanguageFolder> strings (DE, EN, TR), and the default language are defined in local script cfg.js

Hint: 	The default data directory name webiq-designer used in the example, must be adapted, when you configured a different data directory. You can verify your data directory in the “About WebIQ Designer” screen.

Hint: 	Always use lower letters for parent control name and file name to avoid problems while Windows test environment is not case sensitive but ctrlX OS is case sensitive.

## Charts

To display charts in WebIQ, the library plotly.js is used. The following features are implemented:

- Data creation in PLC (number of items variable at runtime)

- Chart updates when data changes

See remarks in PLC example PRG_Chart_Demo() & WebIQ screen Production>Charts for more details.

## Oscilloscope

WebIQ contains a trend element which allows to show trend charts. The smallest recording time is limited by the smallest communication update time. Therefore, with this mechanism you can’t display data which changes in a few msec or even less. With this example you get an oscilloscope functionality, where the PLC program:

- Records data (in every PLC cycle)

- Transfers configuration & data via CSV & JSON strings

- Sends it via OpcUA to WebIQ.

- Communication is like a REST API, via command & status strings

In WebIQ the plotly.js library is used to display the data.

See remarks in PLC example PRG_Osci_Demo(), WebIQ screen Diagnostic>Oscilloscope for more details (e.g. axes titles, name of csv file).

## Teach point table configuration

The package cxVisuals, contains the widget cx-table, which is used to display and edit the teach points. This table is controlled by FB_TeachPointMgr which extends FB_TableMgr. The features are:

- Display/edit any PLC structure array

- Filter/Sort/Clear table columns

- Copy/Paste table rows

- Format table cells via JS functions defined in WebIQ (e.g. 1 => 001)

- Configuration is done within PLC, by adapting the methods in folder Adapt.

The FB provides a manager for arbitrary PLC array of structures. To use the FB for a specific structure, the following must be observed:

- FB_TeachPointMgr

// Init struct array used for table mTable_Init(IdFirst:=TABLE_ID_MIN, IdLast:=TABLE_ID_MAX, Size:=SIZEOF(arTeachPoints[TABLE_ID_MIN])); // Init custom specific data used anywhere in the FB _Robot := Robot; // Call table state machine SUPER^.FB_Main();

- Methods in folder Adapt must be implemented to fit used array of structure and all other features (displayed buttons, confirm button, …). Keep in mind that only the methods which are used by customers program must be created & adapted in FB_ReachPointMgr.

- See description in source code of the methods for usage and examples

## Alarm/Warning/Info Management

We have implemented a multiplexing channel for alarm/warning/info handling with the following features:

- The number of different alarms/warnings/info’s is unlimited (e.g., 1000, 4000, or more)

- Up to 30 alarms can be displayed simultaneously (method mAlarmSet())

- Up to 10 warnings can be displayed simultaneously (method mWarnSet())

- Up to 10 information’s can be displayed simultaneously (method mInfoSet())

- With increasing number of alarms/warnings/info’s, there is no need to add further communication variables

- If the translation not defined in WebIQ, only the error code is displayed

When you do not like this behavior, you can use the default implementation of WebIQ where a single variable for every message item is used.

Alarm IDs starts at 1000. This ensures a correct numeric sort in the alphabetically sorted Localization Manager list of WebIQ. This section shows the steps how to create a new error.

- PLC: In EN_APP_ALARM add a new error enumeration item
e.g. APE_APP_SECURITY_DOOR_OPEN := 1015

- PLC: To raise the error call method FB_Machine_Base.Tools.mAlarmSet().
Beside the error enum you can provide 3 optional strings, which are displayed in HMI.

IF bDoorOpen THEN
    mAlarmSet(enAppError:= APE_APP_SECURITY_DOOR_OPEN, '', '', '');

END_IF

- WebIQ: In Localization Manager create variable alm1015 if not exists and enter the alarm text for all languages.

## Debug> Plc/Python

For demonstration purpose the PLC template can either run the PLC method or any python script. Select mode Manual, to enable the execution of PLC code or python script.

The plc state machine contains a step mode for testing the program. It can be switched on/off while the program is running.

Figure 16: Debug screen

Python scripts can move axes and kinematics and read/write data layer variables. Also, the teach points of this PLC template. An example how to do this, will be shipped in a later version.

The example script only reads/writes variables from the data layer, which are shown in WebIQ HMI:

To use demo HMI page for python script, do the following steps:

- Install IDE & python app on ctrlX OS

- The python scripts are installed. See section 4.1 ctrlX CORE / ctrlX OS

    - DemoDataLayer.py	Read & write of data layer variables, stop script

    - DemoMoveKin.py	Move kinematic & write data layer variables

In WebIQ HMI:

- Select mode Manual or Setup

- Open page Recipe>Setup

  - Use button Text to open textual programs

  - Use button Visual to open visual programs

  - Save the recipe

- Open page Debug>Plc/Python select Run: Python

- “Start” the script in the Automatic Mode which changes the fields bFlag and Info

Figure 17: Debug screen

Hint:	This screen is only a playground.

It is used to demonstrate the start of Python scripts from the PLC. In a customer project the handling of Python scripts must be implemented customer specific.

## Debug> Debug Functions

On this page some debug information’s are displayed. As the configuration of the displayed data takes place entirely in the PLC, no actions are necessary in the HMI project. Especially the table feature is used to display and edit any PLC variable. All base variable types can be displayed.

- To add a new variable, enter it in FB_DebugVarsMgr.mDefineVars()

- Text lists are defined in FB_DebugVarsMgr.mDefineTextList()

- That is all, nothing to do in WebIQ.

Figure 18: example mDefineVars()

## Group rights

On page “Settings> Group rights” you can configure the rights for the user groups.

HINT: 	This framework only provides the functionality for setting group rights. The technical restrictions must therefore be observed when setting the rights. Example: If a feature only works in OP mode setup, it may only be activated in this operating mode. In the other OP modes, where it does not work, it must be deactivated.

The OP modes are displayed in the header. You can select the following options for the OP mode buttons:

| Icon | Meaning | Virtual constant item |
| --- | --- | --- |
|  | button is visible and enabled | .EDH_EDIT=0 |
|  | button is visible and locked | .EDH_LOCKED=3 |
|  | button is invisible | .EDH_HIDDEN=4 |

The main menu items, have the same options as the OP mode buttons.

The sub menu items, have the following options:

| Icon | Meaning | Virtual constant item |
| --- | --- | --- |
|  | Sub menu is selectable. Content of sub menu screen is editable by multiple clients | .EDH_EDIT=0 |
|  | Not yet implemented. Sub menu is selectable. Content of sub menu screen is editable by a single client. The first client which open the screen can edit it. The screen is locked until the client closes the screen | .EDH_EDIT_ONCE=0 |
|  | Sub menu is selectable. Content of sub menu screen is read only. | .EDH_VIEW=2 |
|  | Sub menu is locked | .EDH_LOCKED=3 |
|  | Sub menu is invisible | .EDH_HIDDEN=4 |

The 3rd level of group rights allows to set specific features on a sub menu content page. 
E.g. iMmRecipe/iSmTeachPoints/iRecipeOpen. The same options as for sub menu items are available.

### Features

- Graphical configuration of group rights at runtime

- "Load": Loads default group rights defined in cfg.js (variable jDefaultGroupRights)

- "Save": Save changes and copies content of variable jDefaultGroupRights to clipboard. This code can be copied into cfg.js to update the default group rights.

### Implementation details

In WebIQ code editor open the local script cfg.js, to edit the variables below:

| Action | variable |
| --- | --- |
| Define user groups | const arUserGroups = ["operator", "service", "admin"] User group are displayed in a cx-select widget |
| Define OP modes | const arOpModes = ["iOpSetup","iOpManual","iOpAuto"] Op modes are displayed in header of table |
| Define rights | const arGrpData = [ : ['iMmRecipe', 'gtl-menuMain', 1],// virtual-item, text-list, level ['iSmTeachPoints','gtl-menuSub', 2], // virtual-item: name of virtual item ['iRecipeOpen', 'gtl-menuSub', 3], // text-list: name of text list ['iRecipeEdit', 'gtl-menuSub', 3], // see local script global-textlists ['iSmSetup', 'gtl-menuSub', 2], // level: level of group right ['iSmSafeZones', 'gtl-menuSub', 2], // 1=main menu ['iSmSwitches', 'gtl-menuSub', 2], // 2=sub menu ['iSmVisualProg', 'gtl-menuSub', 2], // 3=features : ] |

- Each right is displayed in a single table row

- Each virtual item, which is defined in arGrpData[] must be created with a dot as prefix. E.g. iMmRecipe => virtual:.iMmRecipe

- Until the virtual items are not used in the WebIQ project, they have no effect

- Example of usage of virtual items:

  - Lock: virtual:.iMmProduction = virtual:EDH_LOCKED

  - Hide: virtual:.iMmProduction = virtual:EDH_HIDDEN

- Adapt the group right text lists items, to the need of customers application, if requested

## Check OpcUa items

PLC variables are repeatedly added, deleted, and changed during program development. This can easily lead to inconsistencies between the HMI and PLC.

This function can be used to check the following:

- Do all OpcUA items exist?

- Has the data type of an item changed?

To start the test, execute the following steps:

- If not logged in, enter name (1) and password (2) of a WebIQ administrator.
(e.g., the user that is used when logging into the WebIQ Designer) and 
press Reload (3) button

- Select OpcUA connection (4)

- Press button (5)

- Wait until the check is finished

- Fix problems

## Debug> Config HMI

The behavior of the HMI project can be set from this page. The settings are stored in the browsers local storage, which means they can be different for every browser/device.

- Support for testing the HMI project
Changing users during testing is time-consuming, as you must log out and log back in. To simplify this, several users can be entered on this page with their password.

If the “Switch User On” option is activated, the following happens:

- After login (or AutoLogin), the system switches to the first user in the list

- When you click on the user icon in the header, a dialog appears that allows you to switch users without entering a password.

Option "Select Menu On"

When this option is activated, the entered "Menu Main" and "Menu Sub" ID is displayed at program start.

- Deactivation of iframe content (e.g.: 3D Viewer) on non-performing HW.
The “Hide iframe content” option is available so that the same WebIQ project runs on high-performance and less powerful devices. If this is activated, alternative content that requires less performance is displayed instead of an iframe.

# WebIQ

## Documentation

WebIQ is a HTML5 based visualization. It works totally different than our previous HMI system WinStudio. If you do not have any HTML5 know how, you should view the videos or read the documentation provided by WebIQ.

Hint: Especially the selection of tabulator panels in the Designer (=sub screens) takes some time to get familiar. At least check this video and in the ctrlX AUTOMATION Community.

| ctrlX AUTOMATION Community | https://developer.community.boschrexroth.com/t5/ctrlX-AUTOMATION-Community/ct-p/dcdev_community |
| --- | --- |
| WebIQ Forum (questions & answers) | https://developer.community.boschrexroth.com/t5/Smart-HMI-WebIQ-Designer-and/bd-p/dcdev_community-dcae-smarthmi |
| WebIQ Documentation | https://docs.webiq.de/docs/webiq-designer-manual/ |
| WebIQ Videos | https://www.smart-hmi.de/dokumentation/ |

## Setup development environment (reverse proxy)

The WebIQ template has some features that only work via a reverse proxy (=caddy). Neither of the two designer previews can be used directly to test the project during development. But there is an easy way to test the WebIQ with all features without publishing it every time.

Open article HowTo-view-content-from-multiple-web-servers-in-one-web-page and follow instructions in section “Install and start caddy”. When you installed caddy you can test the WebIQ project by the link https://localhost:8081/webiq/cx/

Hints:

- After some hours, caddy stops working and it needs to be restarted.

- Enable “Auto Login” in “Hmi Project Settings” of the Designer, to avoid login prompt

## Package Manager

The HMI-Template uses some additional packages, which are included in the project file, for runtime. However, for editing the project, the packages must be installed within the Designer.

- Press (1) to open Package Manager

- Press (2) “Upload package” to select file

- Repeat previous step, for all other packages

Figure 19: WebIQ Designer: Package Manager

## Color Themes

The user can choose between different color themes. The template offers a light Color Theme with Bosch Rexroth’s Corporate Identity Colors. In contrast to that also a dark Theme exists. The two other themes can be fully customized in the HMI settings. The functionality for this theme switching is done by custom code, which you can see in the Code Manager.

In the next chapters the different colors of the themes are listed in a table. The variables in the first column are the variables that are set in the globals.js file. To change the color of the icons the color is inverted.

### Color schema 1 Corporate Design Rexroth

| Variable | Hex Code | Color | Usage |
| --- | --- | --- | --- |
| Functional colors: |  |  |  |
| --col-bg-ok | #78BE20 |  | Messages, Green Leds |
| --col-bg-ok-label | #000000 |  | Label color for ok messages |
| --col-bg-warn | #FCAF17 |  | Used for Warnings |
| --col-bg-warn-label | #000000 |  | Label color for warn messages |
| --col-bg-error | #EA0016 |  | Used for Errors |
| --col-bg-error-label | #000000 |  | Label color for error messages |
| --col-bg-question: | #00CCFF |  | Used for Question Dialog boxes |
| --col-bg-question-label | #000000 |  | Label color for question messages |
| Button colors: |  |  |  |
| --col-bg-btn-normal | #7D7D7D |  | Background color of Buttons |
| --col-bg-btn-active | #00CCFF |  | Background color of Buttons in active state |
| --col-btn-normal-border | #DADADA |  | Border color of Buttons |
| --col-btn-active-border | #91F0FF |  | Border color of Buttons in active state |
| --col-btn-normal-label | # FFFFFF |  | Label color of Buttons |
| --col-btn-active-label | #FFFFFF |  | Label color of Buttons in active state |
| --inv-btn-normal-img | 1 |  | Inverts the icons (0=black,1=white) |
| --inv-btn-active-img | 1 |  | Inverts the icons (0=black,1=white) |
| Background colors: |  |  |  |
| --col-bg-content-window | #FFFFFF |  | Background color of content window |
| --col-bg-content-box | #EFF1F2 |  | Background color of a content box |
| --col-content-font | #000000 |  | Content font color |
| --inv-content-img | 0 |  | Inverts the icons (0=black,1=white) |
| --col-bg-sidebar | #EFF1F2 |  | Background color for the sidebar |
| --col-sidebar-border | #D1DCE3 |  | Color for sidebar border |
| --col-bg-table-hdr | #D1DCE3 |  | Color of the Header of the teachpoint table |
| --col-bg-table-row-active | #7FE5FF |  | Color of the active row in the teachpoint table |
| Header colors: |  |  |  |
| --col-bg-header | #EFF1F2 |  | Background color of the header |
| --col-bg-rexroth-gif | #D1DCE3 |  | Background color of the Rexroth gif |
| --col-bg-header-top | #D1DCE3 |  | Background color of the top header |
| --col-header-label | #000000 |  | Font color of text in the header |
| --inv-header-img | 0 |  | Inverts the icons (0=black,1=white) |
| --col-tab-inactive | #9D9D9D |  | Color for the inactive tabs of the tab-panel |
| --col-tab-inactive-font | #FFFFFF |  | Font color for inactive tabs of the tab-panel |

### Color schema 2 Dark Theme

| Variable | Hex Code | Color | Usage |
| --- | --- | --- | --- |
| Functional colors: |  |  |  |
| --col-bg-ok | #78BE20 |  | Messages, Green Leds |
| --col-bg-ok-label | #000000 |  | Label color for ok messages |
| --col-bg-warn | #FCAF17 |  | Used for Warnings |
| --col-bg-warn-label | #000000 |  | Label color for warn messages |
| --col-bg-error | #EA0016 |  | Used for Errors |
| --col-bg-error-label | #000000 |  | Label color for error messages |
| --col-bg-question: | #00CCFF |  | Used for Question Dialog boxes |
| --col-bg-question-label | #000000 |  | Label color for question messages |
| Button colors: |  |  |  |
| --col-bg-btn-normal | #969696 |  | Background color of Buttons |
| --col-bg-btn-active | #00CCFF |  | Background color of Buttons in active state |
| --col-btn-normal-border | #AAAAAA |  | Border color of Buttons |
| --col-btn-active-border | #91F0FF |  | Border color of Buttons in active state |
| --col-btn-normal-label | # FFFFFF |  | Label color of Buttons |
| --col-btn-active-label | #FFFFFF |  | Label color of Buttons in active state |
| --inv-btn-normal-img | 1 |  | Inverts the icons (0=black,1=white) |
| --inv-btn-active-img | 1 |  | Inverts the icons (0=black,1=white) |
| Background colors: |  |  |  |
| --col-bg-content-window | #121212 |  | Background color of content window |
| --col-bg-content-box | #505050 |  | Background color of a content box |
| --col-content-font | #FAFAFA |  | Content font color |
| --inv-content-img | 1 |  | Inverts the icons (0=black,1=white) |
| --col-bg-sidebar | #646464 |  | Background color for the sidebar |
| --col-sidebar-border | #464646 |  | Color for sidebar border |
| --col-bg-table-hdr | #969696 |  | Color of the Header of the teachpoint table |
| --col-bg-table-row-active | #00CCFF |  | Color of the active row in the techpoint table |
| Header colors: |  |  |  |
| --col-bg-header | #646464 |  | Background color of the header |
| --col-bg-rexroth-gif | #646464 |  | Background color of rexroth gif |
| --col-bg-header-top | #464646 |  | Background color of the top header |
| --col-header-label | #FAFAFA |  | Font color of text in the header |
| --inv-header-img | 1 |  | Inverts the icons (0=black,1=white) |
| --col-tab-inactive-font | #000000 |  | Font color for inactive tabs of the tab-panel |

## Composites

For easy maintenance most of the composite widgets from previous template version are replaced by custom widgets. The custom widget packages will be available soon in the ctrlX AUTOMATION Community with documentation and examples. While the documentation is not available, just check how the custom widgets are used in the template.