## Adapt axis settings

For the commissioning of a real machine, some important information is still missing in the data layer of the ctrlX. As a workaround, this data is saved in the PLC and can be edited in the WebIQ HMI. See Commissioning> Axis.

When the PLC program is started, the saved setting is compared with the existing axes to determine changes. If a deviation is detected, an administrator must adapt the configuration.

### Axis settings

| Label | Meaning |
| --- | --- |
| Axis PROJECT | Name of all axes defined in project |
| Axis CFG | Name of all axes stored in config file |
|  | Displaying both axis name enables you to detect differences between project and stored configuration. If the configuration does not match the project: - Revise settings - Copy axis names with button "Copy name" - Save changes |
| Jog. Rel. | Enables relative Jog in widget cx-jog |
| Ref. Abs. | Enables setting of absolute reference in widget cx-jog |
| Ref. Drive | Enables drive-based referencing in widget cx-jog |
| Ref. Drive Pos. | Show position input field for drive-based referencing. While all parameters for drive-based homing are stored in the drive, sending the position to the drive must be implemented in FB_Machine_Customer.mSetDriveHomingParams() |
| Mode in Auto | OP mode, which is set, when axis does not belong to a kinematic |
| Kin. axis? | Checked if axis is part of any kinematic |

To set other application specific settings, see FB_Machine_Customer/00_Init/mUserCfg.