## Adapt kinematic settings

For the commissioning of a real machine, some important information is still missing in the data layer of the ctrlX. As a workaround, this data is saved in the PLC and can be edited in the WebIQ HMI. See Commissioning> Kinematics.

When the PLC program is started, the saved setting is compared with the existing kinematics to determine changes. If a deviation is detected, an administrator must adapt the configuration.

### Kinematic settings

| Label | Meaning |
| --- | --- |
| KIN_X Min. Pos [] | Min. position of kinematic axis X |
| KIN_Y Min. Pos [] | Min. position of kinematic axis Y |
| KIN_Z Min. Pos [] | Min. position of kinematic axis Z |
| * Max. Pos [] | Max. position of other kinematic axes |
| Jog. Rel. | Enables relative Jog in widget cx-jog |
| Max. Jog Vel. [] | Max. jog velocity for kinematic |

To set other application specific settings, see FB_Machine_Customer/00_Init/mUserCfg.