## Plc/Python

For demonstration purpose the PLC template can either run the PLC method or any python script. Select mode Manual, to enable the execution of PLC code or python script.

The plc state machine contains a step mode for testing the program. It can be switched on/off while the program is running.

Figure 1: Debug screen

Python scripts can move axes and kinematics and read/write data layer variables. Also, the teach points of this PLC template. An example how to do this, will be shipped in a later version.

The example script only reads/writes variables from the data layer, which are shown in WebIQ HMI:

To use demo HMI page for python script, do the following steps:

- Install IDE & python app on ctrlX OS

- The python scripts are installed. See section Fehler! Verweisquelle konnte nicht gefunden werden. Fehler! Verweisquelle konnte nicht gefunden werden. / ctrlX OS

    - DemoDataLayer.py	Read & write of data layer variables, stop script

    - DemoMoveKin.py	Move kinematic & write data layer variables

In WebIQ HMI:

- Select mode Manual or Setup

- Open page Recipe>Setup

  - Use button Text to open textual programs

  - Use button Visual to open visual programs

  - Save the recipe

- Open page Debug>Plc/Python select Run: Python

- “Start” the script in the Automatic Mode which changes the fields bFlag and Info

Figure 2: Debug screen

Hint:	This screen is only a playground.

It is used to demonstrate the start of Python scripts from the PLC. In a customer project the handling of Python scripts must be implemented customer specific